COMPLIANCE  STATUS  ON THE  CONDITIONS OF ENVIRONMENTAL CLEARANCE FOR THE EURO III MOTOR SPIRIT PROJECT OBTAINED VIDE LETTER  NO.  J-11011/92/2003 – IA II  (I)  DATED FEBRUARY 13,

2004  FROM MOEF, NEW DELHI

A.SPECIFIC CONDITIONS:

i.	The company shall ensure strict implementation / compliance of the terms and conditions mentioned vide Ministry’s letter No. J-11011/16/90-IA.II dated 31.05.1991.

-   Noted.

ii.	The company shall ensure that the total sulphur emission from the Assam refinery (including MS Quality Improvement Project) shall not exceed 128 kg/hr as sulphur (256 kg/hr as SO2). M/s NRL should maintain regular record of sulphur balance in the refinery. Off gases from the proposed unit should be treated in amine absorption and regeneration unit meant for H2S removal for desulphurization of off gases. Performance evaluation of sulphur recovery block should be done regularly. Data on VOC should be monitored and submitted to the Ministry. The continuous emission monitoring systems for SOx and NOx in the major stacks with proper calibration facilities should be installed. The low NOx burners should be installed in all the furnaces.

-The total sulphur emission from the refinery including Euro III Motor Spirit Project shall be maintained below 128 kg/hr as Sulphur (256 kg/hr as SO2).

-Regular sulphur balance of the refinery is maintained and the average SO2 emission from the refinery during current year from April’16 to Sept’16 is   91.9  kg/hr only.

-Off gases from the proposed unit has been treated in the amine absorption and regeneration unit.

-Performance evaluation of Sulphur Recovery Block is done on a daily basis.

-The data on VOC has been monitored and submitted to the Ministry.

-Continuous emission monitoring for SO2 and NOx have been provided in all

the stacks.

-Ultra low NOx burners have been provided in all the furnaces.

iii.		Additional water requirement shall not exceed 1200 m3/hr. The total quantity of effluent generation should not exceed 3830 m3/day as indicated in the EMP of which (3530 m3/d from the existing and 300 m3/d from the proposed unit). Treated effluent should be recycled and rest should be discharged after primary, secondary and tertiary treatment into the Dhansiri river through 11 km long pipeline. The treated effluent should comply with the prescribed standards.

-Additional water requirement and treated effluent discharged is maintained within the limits. The treated effluent quality is maintained within the prescribed standards and no treated effluent is discharged from the refinery into River Dhansiri since October,2006. Further no treated effluent from township is discharged since April, 2007 which means the effluent is totally recycled.

iv.    As reflected in the EIA / EMP, the spent catalyst (0.33 TPM) along with small quantity of oily and chemical sludge should be disposed off in secured landfill site within the plant premises. The leachate from the landfill site should be sent back to the effluent treatment plant. The ground water quality around the secured landfill site should be monitored regularly and data submitted to the Ministry/CPCB/SPCB. The biological sludge generated from the ETP should be used as manure/fertilizer for the green belt.

The oily sludge, generated during the cleaning of crude tank post BLABO process has been disposed off through bioremediation. Oily sludge generated in ETP is disposed off in the Secured Land Fill  after meeting the requiremet. As per design of the Secured Land Fill, the leachate generated is routed back to the IRS of ETP  for further processing.

The  ground  water quality  monitoring  around  the  Secured Landfill site is  being done on a regular basis. The monitoring report has been  submitted  regularly to the MoE&F Regional Office, Shillong along with the half-yearly report and to the CPCB and SPCB.

Ground water monitoring data from Apr’16- Sept’16 around Secured Land Fill is enclosed as Annexure –I

v.	Oil spill response facilities should be in place, in accordance with OISD guidelines with regard to the likely risks associated with transportation of finished products. All recommendations made in the risk analysis report should be complied with during design, construction and operation stages to contain the risk within the plant boundary.

- The same is complied.

- Oil from various units is routed through CRWS (Contaminated Rain Water System) to ETP. The contaminated rain water from various units, OM&S and NRMT go through the CRW system to IRS (Inlet Receiving Sump) in ETP. The contaminated Oil is processed in ETP and sent through slop line to OM&S.

The Storm Water Channel from various plants are connected and routed through Oil Catchers. There are five oil catchers in the final outlet of Storm Water channel. The final outlet of storm water channel is closed immediately if oil flows into the oil catchers. By operating Turbo pum, oil is delivered to OWS. The accumulated oil from the oil catchers is lifted with the help of MOSRU (Mobile Oil Spill Recovery Unit).  The recovered oil is dispatched to ETP through OWS line and in IRS by Gully sucker.

vi.	Green Belt of adequate width and density as per the CPCB guidelines should be provided to mitigate the effects of fugitive emission all around the plant in consultation with the local DFO. The bio sludge should be used as manure in the Green Belt development.

A Green Belt of width around 100 mtrs surrounding the refinery and around 25 mtrs. around the NRMT covering a total area of about 60 hectares have been provided. The bio sludge is used as manure in the gardens.

vii.	Occupational Health Surveillance of the workers should be done on a regular basis and records maintained as per the Factories Act and the West Bengal Factories Rules.

- 	Occupational Health Surveillance of the workers has been done on a regular basis and records are maintained properly.

B.	GENERAL CONDITIONS:

i.	The project authorities must strictly adhere to the stipulations made by the Assam Pollution Control Board and the State Government.

-The stipulations made by the Assam Pollution Control Board and the State Government are strictly adhered to.

- No further expansion or modernization in the plant should be carried out without prior approval of the Ministry of Environment and Forests.

- Noted., any expansion or modernization shall be carried out with the proper  approval of Ministry only.

- The Company shall implement all recommendations made in the EMP and Risk Analysis reports.

-	Complied.

- At no time, the emissions should go beyond the prescribed standards. In the event of failure of any pollution control system adopted by the units, the respective unit should be immediately put out of operation and should not be restarted until the desired efficiency has been achieved.

- The same has been noted.

v.	The overall noise levels in and around the plant area should be kept well within the standards (85 dBA) by providing noise control measures including

acoustic hoods, silencers, enclosures etc. on all sources of noise generation. The ambient noise levels should conform to the standards prescribed under EPA Rules, 1989 viz. 75 dBA (day time) and 70 dBA (night time).

-	The overall noise levels in and around the plant premises are being be maintained as per the requirement. For the same, control measures like silencer to vent, low noise Rotary equipment have been provided. The ambient noise levels all around the refinery are monitored regularly so as to maintain the standards prescribed under EPA Rules, 1989 viz. 75 dBA (day time) and 70 dBA (night time).

-Noise monitoring result carried out recently is enclosed as Annexure II

- The project authorities must strictly comply with the rules and regulations under Manufacture, Storage and Import of Hazardous Chemicals Rules, 1989 as amended in 1994 and 2000. Prior approvals from Chief Inspectorate of Factories, Chief Controller of Explosives, Fire Safety Inspectorate etc. must be obtained.

The rules and regulations under the Manufacture, Storage and Import of Hazardous Chemicals Rules, 1989 and as amended in 1994 and 2000 are adhered to. Approvals from Chief Inspectorate of Factories, Chief Controller of Explosives etc as applicable for the proposed unit have been obtained.

vii.	The project authorities must strictly comply with the rules and regulations with regard to handling and disposal of hazardous wastes in accordance with the Hazardous Wastes (Management and Handling) Rules, 2003. Authorization from the State Pollution Control Board must be obtained for collection/treatment/storage/disposal of hazardous wastes.

The rules and regulations with regard to handling and disposal of hazardous wastes in accordance with the Hazardous Wastes (Management, handling & Transboundary Movement) Rules, 2008 with latest amendments are adhered to. In regard to the same, authorization for collection/treatment/storage and disposal of hazardous wastes through Secured Land Fill has been obtained from the PCB, Assam.

viii.	The project authorities will provide adequate funds both recurring and non-        recurring to implement the conditions stipulated by the Ministry of Environment & Forests as well as the State Government along with the implementation schedule for all the conditions stipulated herein. The funds so provided should not be diverted for any other purposes.

- Adequate fund has been provided for implementing the conditions stipulated by the MoEF and the State Govt and shall not be diverted for any other purpose.

ix.  The stipulated conditions will be monitored by  the Regional Office of this

Ministry at Shillong / Central Pollution Control Board/The State Pollution

Control  Board.   A  six monthly compliance report and the monitored data

should be submitted to them regularly.

-		A six monthly compliance report on the Environmental Clearance conditions and NOC conditions of Numaligarh Refinery including the compliance status on the environmental Clearance for CCU and MS Plant has bee submitted six monthly regularly to the MoEFCC Regional Office, Shillong, CPCB, Shillong and the SPCB, Regional Office, Golaghat, Assam and  the same are uploaded regularly in company’s website also.

x.		The Project Proponent should inform the public that the project has been accorded environmental clearance by the Ministry and copies of the clearance letter are available with the State Pollution Control Board / Committee and may also be seen at Website of the Ministry of Environment & Forests at http:/envfor.nic.in. This should be advertised within seven days from the date of issue of the clearance letter, at least in two local newspapers that are widely circulated in the region of which one shall be in the vernacular language of the locality concerned and a copy of the same should be forwarded to the Regional office.

The same has been complied. Advertisement regarding the environmental clearance for the Euro III MS Project was published in two local newspapers, The Assam Tribune (in English) and The Pratidin (in Assamese) on 18th Feb'04 and copies of both News Paper were forwarded to the MoE&F Regional Office, Shillong vide letter no NRL/NG/ENV/2.1/11 dated 20th Feb'04.

xi.	The Project Authorities should inform the Regional Office as well as the Ministry, the date of financial closure and final approval of the project by the concerned authorities and the date of commencing the land development work.

-The same has been complied.

- The Ministry may revoke or suspend the clearance, if implementation of any of the above conditions is not satisfactory.

- Noted.

4.0	The Ministry reserves the right to stipulate additional conditions if found necessary. The Company in a time bound manner will implement these conditions.

- Noted.

5.0	The above conditions will be enforced, inter-alia under the provisions of the Water (Prevention & Control of Pollution) Act, 1974, the Air (Prevention & Control of Pollution) Act, 1981, the Environment (Protection) Act, 1986, Hazardous Waste (Management, Handling & Transboundary Movement) Rules, 2008 and the Public Liability Insurance Act, 1991 along with their amendments and rules.

- Noted.

********