COMPLIANCE STATUS ON THE CONDITIONS OF THE ENVIRONMENTAL CLEARANCE FOR NUMALIGARH REFINERY OBTAINED VIDE LETTER NO. J-11011/16/90-IA-II DATED MAY 31st '1991

FROM MOEF, GOVT. OF INDIA

- The layout of the refinery should be so planned within the proposed site so as to ensure that it is situated as far to the eastern side of the site as possible, to ensure that there is the maximum possible distance from the eastern boundary of the Kaziranga National Park. The layout of the site of refinery may be finalised in consultation with this Ministry.

-	Complied.

- The residential site as proposed should not be to the west of the refinery as it is only 19.5 kms from the boundary of Kaziranga National Park. It should be shifted further away, but keeping in view the distance from the Garampani Sanctuary, which is only 24 kms south of the proposed refinery site. The newly proposed site of the residential colony should be settled to the satisfaction of this Ministry.

- Complied.

- The National Highway-37 should be diverted away from the Kaziranga National Park and that portion of this road through and along the National Park (From Jakhalabandha to Bokakhat) to be denotified from all highway  records  and  handed  over to the National Park Authorities for regulating traffic. No movement of personnel, material or equipment for the project shall take place on the existing National Highway-37. The realignment of the National Highway-37 would be finalised in consultation with the Ministry of Environment and Forests, so that the wildlife habitat in the nearby Mikir Hills and areas rich in biological diversity therein are protected. Work on the diversion of NH-37 will start before construction of the refinery begins and the Ministry of Petroleum should ensure that the road is completed before the commissioning of the project.

-  Matter has been taken up with NHAI New Delhi & Guwahati Office. The same has also been appraised to MoEF.  NRL has requested for waival of this clause, as these conditions do not fall under the purview of NRL.

- A No Development Zone must be notified before the project construction starts within a radius of 15 kms all around the refinery site, except towards the northwest where the no development zone would extend into the Eastern boundary of the Kaziranga National Park.

-	The Govt. of Assam has already notified the “No Development Zone” on 19.01.95.

- No pipeline will be laid through the Kaziranga National Park and adjacent wildlife habitats in the Mikir Hills. Pipeline alignment shall be finalized in consultation with the Ministry of Environment and Forests to minimize impact on environment and forests.

-	NRL has not laid any pipeline through Kaziranga National Park.

- The project authority must strictly adhere to the stipulations made by the SPCB and the State Government.

-	The stipulations laid down by SPCB, Assam and the State Govt. are adhered to. The status on the compliance report has been regularly sent to PCBA, Regional Office, Golaghat. A copy is enclosed as Annexure B.

- Any expansion of the plant either with the existing product mix or new products can be taken up only with the prior approval of this Ministry.

-	This is complied with as and when a change or expansion is contemplated. Any expansion of the plant either with the existing product mix or new products will be taken up with prior approval of the Ministry.

- The gaseous emissions from various process units should conform to the standard prescribed by the concerned authorities, from time to time. At no time, the level should go beyond the stipulated standards. In the event of failure of any pollution control system adopted by the unit, the respective unit should be put out of operation immediately and should not be restarted until the control measures are rectified to achieve the desired efficiency.

-	Automatic online stack analysers have been provided in all the major stacks for continuous monitoring of SO2 and NOx. Further manual stack monitoring is being carried out bi-monthly as per latest MOEF notification. Monitoring reports of stack emissions are regularly submitted to the PCBA Regional Office, Golaghat in every month.

- A minimum of five air quality monitoring stations should be set up in the downwind direction as well as where maximum ground level conc. is anticipated. Furthermore, stack emission should be monitored by setting up of automatic stack monitoring units. The data on stack emission should be submitted to the SPCB once in three months and to this Ministry once in six months, along with the statistical analysis. The air quality monitoring stations should be selected on the basis of modeling exercise to represent the short-term ground level conc.

- As an action of compliance,  five (5) nos. of ambient air quality monitoring stations have been set up at the following locations:

SS 1	:      Inside the refinery (Near WT No.6).

SS 2	:      At the Eco-Park in NRL Township.

# SS 3	:      At the Raw Water Intake.

# SS 4	:      At Rongbong near the NH-39 bypass.

SS 5	:      At Agartoli near the Kaziranga Wildlife Sanctuary.

- Ambient Air Quality monitoring at the above locations has been carried out regularly in line with NAAQS-2009 in totality.  The Ambient Air Quality Monitoring reports are regularly submitted to the PCBA HQ Guwahati, PCBA Regional Office, Golaghat. Online realtime emission data has been transmitted to CPCB server.

-	Automatic online stack analysers have been provided in all the major Stacks for continuous monitoring of SO2 and NOx. The monitoring reports of stacks emissions are regularly   submitted to the PCBA Regional Office, Golaghat and  in every month and to the MoE&F Regional Office, Shillong& CPCB Regional Office  in every six month.

NRL has installed one continuous Ambient Air Monitoring System inside the refinery premises during 2008-09. The online data is being displayed in the LED display board installed near the refinery gate. Further. Further, installation of additional analysers like PM2.5, O3, C6H6 and NH3 as per NAAQS 2009.have been completed . Realtime emission data has been transmitted to CPCB server.

As advised by MoEFCC, NRL is planning to install one new CAAQMS in a suitable location inside the refinery premises in the downwind direction anticipating the maximum ground level concentration.

-Ambient air quality for the period Aprt’16 to Sept’16 is enclosed as annexure-IV.

- There should be no change in the stack design without the approval of SPCB. Alternate Pollution control system and proper design (Steam Injection System) in the stack should be provided to take care of excess emissions due to failure in any system of the plant.

-Prior approval of SPCB will be taken for any change in the stacks design. Pollution control measures like – Low NOx burners, Steam Injection System, Low excess air firing, ID and FD fan, Stack dampers have been provided.

- Only natural gas after de-sulphurization has to be used as fuel with low NOX burners.

-	Permission has been obtained from MoE&F, New Delhi vide No. J-11011/16/90-IA.II(I) dated 22 May,1996 to use Naphtha as fuel in CPP, which is produced in the refinery itself. Low NOx burners have been installed in all the refinery furnaces. From March’ 2011onwards , natural gas has been provided  as fuel in CPP and as feed in Hydrogen Unit as a partial replacement to Naphtha.

- Fugitive emissions should be monitored continuously.

- Regular monitoring of fugitive emission has been carried out using GMI since May, 2005 onwards in all the process units, tank firm areas etc.

The GMI survey has been carried on all gas/vapour valves, light liquid valves, hydrogen valves, light liquid pump seals, hydrocarbon compressor seals, hydrogen compressor seals, safety relief valves, flanges, connections, open-ended lines, drains, tankages, furnaces etc.

Further, as a step towards monitoring and control of fugitive emission from storage tanks, phasewise installation of double seal is in progress in both EFRT & IFRT tanks. Also, a VOC recovery system in ETP has been installed.

- All gaseous emissions in the system shall be taken to the flare system and the flare should be smoke-less and non-luminous.

-	All gaseous emissions have been taken to the flare system. A non-luminous elevated flare has been installed as regular flare. However, additionally, a ground flare has been also installed for using during emergencies. A Flare gas recovery system is also under implementation.

- A sulphur recovery plant should be commissioned along with the refinery.

- The Sulphur Recovery Block (SRB) has been commissioned alongwith the refinery and has been under continuous operation since September’2000.

- Zero discharge of effluents should be ensured and built into the system. In case the effluent has to be discharged due to process disturbances etc. the contributing unit shall be immediately stopped from operation and will not be re-started without bringing the system to normalcy. To meet the emergency needs adequate number of effluent quality monitoring stations must be set up in consultation with the SPCB.

- Since October, 2006 no effluent from refinery has been discharged into the River Dhansiri and since April, 2007 township effluent also routed to the refinery, so no effluent from refinery & township is discharged outside and the total effluent is recycled and reused within the Refinery as Fire water makeup, watering in Green Belt and filter back washing in Cooling Tower.

Treated effluent quality for the period Aprt’16 to Sept’16 is enclosed as annexure-V.

- Guard ponds of sufficient holding capacity to take care of monsoon rains should be provided.

-	Guard ponds (of capacity: 5329 m3) for oily water sewer (OWS) and Surge tank (of capacity: 5760 m3) for contaminated rain water system (CRWS) have been provided in the Effluent Treatment Plant.

- The solid waste from the ETP and waxy sludge should be incinerated.

- Oily sludge generated in ETP has been disposed off in the Secured Landfill facility after meeting the criteria.  Other oily sludge generated from the cleaning of crude tank post BLABO process is disposed off thorough Bio-remediation.

- The solid waste (other than waxy sludge) dumping area should be made impervious so that the ground water, is not affected due to leaching and seepage of associated water containing pollutants. The solid waste disposal plan should be submitted to the Ministry once the process design and technological package is finalized.

-As per the recommendation of NEERI’s report on Solid Waste Management, scheme for disposal of solid waste through Secured Landfill had been prepared. As compliance of the same a Secured Landfill facility was constructed within the Refinery premises and commissioned on 4th March, 2004.

The proposal of solid waste disposal by Secured Land Fill was submitted to MoE&F and PCB, Assam.

NRL has constructed  one new Secured Land Fill Facility having a capacity of 6000 m3 as per the latest CPCB guidelines with proper leachate collection and treatment facility.

- The project authorities should recycle the waste to the maximum extent and the recycling plan should be submitted along with a comprehensive EIA.

- Since inception, Numaligarh Refinery has adopted in-built measures to minimize and control pollution and generation of waste in all the units with proper collection and disposal system.

- Adequate segregation, collection and treatment facilities for wastewater for centralized treatment have been provided to meet the stringent standards laid down in the latest MoEF Notification. Therefore, general of hazardous waste and oily sludge at Numaligarh Refinery is kept at minimum.

- However, a small amount of wastes generated from the refinery are recycled to the maximum extent possible. -The recycling plan for all types of wastes have been submitted to the MoE&F, Shillong vide letter no. NRL/NG/ENV/2.1/2 dated May’23, 2002.

- A detailed risk-analysis based on Maximum Credible Accident analysis should be submitted once the process design and the layout are frozen. Based on this a disaster management   plan   has to be prepared and after approval by the concerned nodal agency, should be submitted to this Ministry.

-	NEERI was engaged for Quantitative Risk Assessment including Maximum Credible Accident (MCA)analysis, Hazard Assessment and Evaluation,  Disaster Management Plan (DMP) and Emergency Preparedness Plan(EPP).The report was submitted to the MoE&F on 03.02.97.

- A comprehensive EIA report covering one year (4 seasons) data should be submitted once the process design and technology package and layout are frozen. The Ministry or any other competent authority may stipulate any further conditions after reviewing the comprehensive impact assessment report.

-	CEIA report prepared by NEERI has been submitted to MoE&F on 22.04.96.

- A comprehensive study of the ecological status and likely impact of development should be initiated in consultation with the Ministry of Environment and Forests. The proposal should be submitted to this Ministry within three months.

-	This is included in the CEIA report and submitted.

- A green belt with a minimum width of 500 mts should be provided and the green belt development plan taking into account various aspects including attenuation of noise and air pollution should be submitted to this Ministry within six months.

-Initially, as per Environmental Clearance granted for the Numaligarh Refinery        Project, Ministry of Environment & Forest had stipulated a 500 mtrs wide green         belt all around the refinery based on the EIA of Numaligarh Refinery carried out         by NEERI.

On request from Numaligarh Refinery, the width of the Green Belt was later         reduced from the suggested width of 500 mtrs to 100 mtrs  because of  the reason        that almost all the surrounding areas  are  having  tea  garden  with shade trees       (Sirish trees).A wide natural green belt already existed all around the refinery.

Accordingly, a Green Belt covering a total area of around 60 hectares of land and

around 100 mtrs width around the refinery and around  25 mtrs  width  around the  NRMT has been developed as per the Green Belt Development Plan. (The Green Belt Development Plan has been submitted to MoEF along with the Half  Yearly  Report to MOEF on the 15th October, 2001).

Massive Plantation have been carried out in the Green Belt so as to it can provide

a natural barrier for  attenuation of noise and air pollution.  Nos of   local variety

have been planted including some fruit bearing samplings in  &  all  around  Green Belt. Further, an enumeration porgramme is also carried out in the greenbelt area   for proper identification of the planted plants.

Again it has been Planned to increase the density by planting more saplings in the

Green Belt in the days ahead.

24. The rehabilitation plan for the persons to be displaced from the project site including

township should be prepared and  submitted  to the  Ministry for  approval  within 3

months. Plan should inter-alia contain rehabilitation site details, the facilities and

compensation package to be provided.

-Rehabilitation of the displaced persons from the refinery site has been done by the State Govt. NRL has paid the compensation as fixed by Collector. Similar action has  also been taken in case of those displaced from the Township site for which land acquired. Amenities like periodical medical check-up facility, drinking water facility, building primary school, constructing roads in the villages etc. have been contributed by NRL.

25.The project authority must set up a laboratory facility for collection and analysis of

samples under the supervision of competent technical person, who will directly report

to the Chief Executive.

-The laboratory facility had been set up for collection and analysis of samples under the supervision of competent personnel, reporting to the Chief Manager (QC)and who reports to the General Manager (Technical). The NRL QC laboratory is accredited by NABL.

26. A separate environment management cell  with suitably qualified people to carry out

various  functions  should  be set up under the control of senior executive who will

report directly to the head of the organization.

- A fully functional, dedicated environment management cell manned by qualified engineers/officers and headed by Deputy General Manager (Technical) has been  continuously working for constant improvement, monitoring, safe guarding and reporting of environmental activities of the refinery. Also, a multidisciplinary Apex-level Committee on Environment which includes senior level officers from various departments as members under the chairmanship of Director (Technical) constantly guides the Environment Cell regarding all the environmental issues in the refinery. The Apex Committee that convenes quarterly discusses the unresolved issues if any, regarding the environment and monitors the regular environmental activities.

27.	The funds ear-marked for the environmental protection measures should not be

diverted for other purposes and year-wise expenditure should be reported to this

Ministry.

-	 The same has been complied with.

a).	The Ministry may revoke clearance if implementation of the conditions is  not satisfactory.

-	 Noted

b).    The above conditions will be enforced inter-alia under the provisions of the    Water (Prevention & Pollution) Act, 1981, and Environment (Protection) Act, 1985 along with their amendments.

-   Noted

**********

Annexure-A

CPMPLIANCE STSTUS OF THE SIX CONDITIONS GIVEN WITH THE DEVELOPMENT OF TOWNSHIP OBTAINED VIDE LETTER NO. J-11014/2/91-IA.II DATED 18TH JANUARY, 1994 FROM MOEF NEW DELHI.

_______________________________________________________________________

CONDITIONS:

i)   The hill slopes should not be used for civil construction purposes

- Complied , the hill slopes have not been used for civil construction purposes

ii) Land use planning of the colony and the land around it should be finalized in construction with the State Town Planning Department.

- Consultation was done, but minutes of the same are not available at present.

iii) More open space should be left and the building construction may be done by acquiring minimum land and the houses should be constructed on ground plus two floors basis.

Complied, open space left within the township is around is around 82% of the total

area.Building were constructed as per direction as above.

iv) Township site should not involve any forest area.

-    Township is constructed only in the permitted area.

v)  The existing forest cover towards the west and north of the proposed colony site should not be disturbed.

- The same has  not disturbed.

vi) No organized human settlement or private colonies should be allowed in the hill or the areas adjoining the hill. (Atleast in a radius of 10 kms).

-     The Govt. of Assam has already notified the “No Development Zone” on 19.01.95.

**********